MEHRAN UNIVERSITY OF ENGINEERING & TECHNOLOGY JAMSHORO DEPARTMENT OF ELECTRONIC ENGINEERING COURSE:

FPGA BASED DIGITAL DESIGN (PR) BATCH =20ES, 6TH SEMESTER

COURSE TEACHER: ENGR. SHOAIB HASSAN

Submitted By: 20ES062

PBL ACTIVITY

Instructions

- The solution must be submitted by each student individually.

- The answers should be submitted as printed hardcopy.

- The softcopy of the report should also be submitted on MS Team.

- All references to books, online resources, lectures, and other material must be properly included in the report.

PBL activity Description: Design a system using NEXYS4 DDR FPGA board which performs all the mentioned Arithmetic, Logic, Shift, rotate and relational functions given in table 1 between two signals A(3:0) and B(3:0).

The block diagram of the system is shown in the following figure 1.

Use all eight 7-segment displays of the FPGA board and display the outputs as given table 2.

# Figure 1.

PBL Tasks

- Attach the VHDL code in structural Style for designing this system with description of the code.

- Simulate the system by writing Test Bench.

- Attach the simulation result and hardware results snapshots.

# Table 1.

| Select Input (S) | Operation | Function | Unit |
| --- | --- | --- | --- |
| 00000 | Y<= B | Transfer B | Arithmetic |
| 00001 00010 00011 00100 | Y<= A Y<= B+1 Y<= B-1 Y<= A+1 | Transfer A Increment B Decrement B Increment A |  |
| 00101 | Y<= A-1 | Decrement A |  |
| 00110 | Y<= A+B | Add A and B |  |
| 00111 | Y<= B-A | Subtract A from B |  |
| 01000 | Y<=A and X“00” | Clear A | Set and Clear |
| 01001 01010 01011 01100 | Y<= B xor X”FF” Y<=A OR X“FF” Y<=B and X“00” Y<= A xor X“FF” | Toggle B Set A Clear B Toggle A |  |
| 01101 | Y<=A OR X “FF” | Set B |  |
| 01110 | Y<= inp2 & inp1 | Concatenate inp2 and inp1 |  |

| 01111 | Y<= Y | No change |  |
| --- | --- | --- | --- |
| 10000 | Y<= NOT B | Complement B | Logic |
| 10001 10010 10011 | Y<= NOT A Y<= A AND B Y<= A OR B | Complement A AND OR |  |
| 10100 | Y<= A NAND B | NAND |  |
| 10101 | Y<= A NOR B | NOR |  |
| 10110 | Y<= A XOR B | XOR |  |
| 10111 | Y<= A XNOR B | XNOR |  |
| 11000 | Y<= A SLL 2 | A logical left shift by 2 | Shift & rotate |
| 11001 11010 11011 11100 | Y<= B SRL 2 Y<= B SLA 3 Y<= A SRA 1 Y<= B ROR 2 | B logical right shift by 2 B arithmetic left shift by 2 B arithmetic right shift by 2 B rotate right by 2 |  |
| 11101 | Y<= A ROL 4 | A Rotate Left by 2 |  |
| 11101 | Y<= A SLL -2 | A logical left shift by -2 |  |
| 11111 | Y<= B SRL -2 | B logical right shift by -2 |  |

Table 2.

| OUTPUTS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S.No. | Display8 | Display7 | Display6 | Display5 | Display4 | Display3 | Display2 | Display1 |
| 1. | d | E | P | Ւ | E | L | E | C |
| 2. | b | A | Ւ | C | h | - | 2 | 0 |
| 3. | г | O | L | L | n | O | R1 | R2 |
| 4. | г | E | S | Ւ | - | Y-digit3 | Y-digit2 | Y-digit1 |

Note: For Shift and rotate operations, use Concatenation Operator (&).

Where R1 and R2 represent LSD and MSD of your Roll Number, respectively. In the last row of the table above, Y- digit1 to Y-digit3 must be in decimal format. The results in the table will change every 10 seconds.

# Assessment Rubrics

| Performance Indicators | Marks | PLO Mapping | Good | Average | Poor | Secured Marks |
| --- | --- | --- | --- | --- | --- | --- |
| Problem Recognition | 0.5 | PLO-5 | Demonstrates the ability to identify problems | Demonstrates the ability to identify problems with some assistance | Not able to identify any problems. |  |

| Innovation | 0.8 | PLO-5 | Demonstrates an Innovative idea and successfully implements it | Demonstrates an Innovative idea but doesn’t know how to implement it | Fails to give any innovative idea |  |
| --- | --- | --- | --- | --- | --- | --- |
| Use of Modern Engineering Tools | 0.7 | PLO-5 | Hardware/software are extensively used in the course | Hardware/software are somewhat utilized, effort was put into learning new software | Hardware/softw are are not utilized, no attempt was made at learning new software |  |
| Calculation and Coding | 01 | PLO-5 | Calculation / Coding is complete and details are provided. | Calculation / Coding is complete but details are missing. | Calculation / Coding is incomplete |  |
| Observation/ Program results | 01 | PLO-5 | Observations / program results are complete and details are provided. | Observations / program results are complete but details are missing. | Observations / Program results are incomplete. |  |
| Teamwork | 01 | PLO-10 | Actively engages and cooperates with other group members in an effective manner. | Cooperates with other group members in a reasonable manner. | Distracts or discourages other group members from conducting the experiment. |  |

Submitted By 20ES070

VHDL CODE:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL; use IEEE.NUMERIC_STD.ALL;

entity PBL_070 is

Port ( Clk : in STD_LOGIC;

in1 : in STD_LOGIC_VECTOR (3 downto 0); in2 : in STD_LOGIC_VECTOR (3 downto 0); Sel : in STD_LOGIC_VECTOR (4 downto 0);

Anode : out STD_LOGIC_VECTOR (7 downto 0);

Display : out STD_LOGIC_VECTOR (7 downto 0)); end PBL_070;

architecture USAMA of PBL_070 is

CONSTANT R1: std_logic_vector(3 DOWNTO 0):= "0111"; ---7

CONSTANT R2: std_logic_vector(3 DOWNTO 0):= "0000"; ---0 Signal A, B : std_logic_vector(7 downto 0);

Signal logic,Shift_rotate,Set_clear : std_logic_vector(7 downto 0); Signal Arith : Signed(7 downto 0);

SIGNAL A_sig, B_sig: SIGNED (7 DOWNTO 0 );

Signal Slow : std_logic; SIGNAL fast : std_logic;

Signal Temp ,sevensegcounter : std_logic_vector(3 downto 0); SIGNAL y_logicdecimal: integer range 0 to 255;

SIGNAL y_arithdecimal: integer;

SIGNAL y_SRdecimal: integer range 0 to 255; SIGNAL y_SCdecimal: integer range 0 to 255; SIGNAL ydigit1: integer;

SIGNAL ydigit2: integer; SIGNAL ydigit3: integer;

SIGNAL ydigit1conv,ydigit2conv,ydigit3conv: std_logic_vector(3 downto 0); SIGNAL y1,y2,y3: STD_LOGIC_VECTOR(7 DOWNTO 0);

signal roll, batch, dept : std_logic; begin

process(in1,in2) begin

A<= in1 & "0110"; B<=in2 & "0011";

end process;

----LOGIC UNIT---

With Sel(2 downto 0) Select Logic <= (not B) when "000",

(not A) when "001",

(A and B) when "010", (A or B) when "011",

(A nand B) when "100",

(A nor B) when "101", (A xor B) when "110",

(A xnor B) when others;

----ARITHMETIC UNIT--- A_sig <= SIGNED(A); B_sig <= SIGNED(B);

With Sel(2 downto 0) Select Arith<= B_sig when "000",

A_sig when "001",

(B_sig + 1) when "010",

(B_sig - 1) when "011",

(A_sig + 1) when "100",

(A_sig - 1) when "101", (A_sig + B_sig) when "110",

(B_sig - A_sig) when others;

----SHIFT & ROTATE UNIT---

With Sel(2 downto 0) Select

Shift_rotate <= (A(5 downto 0) & "00") when "000",--A logical left shift by 2 ("00" & B(7 downto 2)) when "001", --B logical right shift by 2

(B(4 downto 0) & B(0) & B(0) & B(0)) when "010", --B arithmetic left shift by 3 (A(7) & A(7 downto 1)) when "011", --A arithmetic right shift by 1

(B(1 downto 0) & B(7 downto 2)) when "100", --B rotate right by 2 (A(3 downto 0) & A(7 downto 4)) when "101", --A Rotate Left by 4

("00" & A(7 downto 2)) when "110", --A logical left shift by -2

(B(5 downto 0)& "00") when others; --B logical right shift by -2

----SET & CLEAR UNIT---

With Sel(2 downto 0) Select

Set_clear <= (A and X"00") when "000",--Clear A (B xor X"FF") when "001",--Toggle B

(A or X"FF") when "010", --Set A

(B and X"00") when "011", --Clear B (A xor X"FF") when "100", --Toggle A (A or X"FF") when "101", --Set B

(in2 & in1) when "110", --Concatenate inp2 and inp1 set_clear when others; --No change

------FREQ DIVIDER (FOR 100KHZ):---------------

process(clk,slow)

variable divider: integer range 0 to 500; begin

if(clk'event and clk='1') then divider:=divider+1;

if(divider=500) then slow<= not slow; divider:=0;

end if; end if;

end process;

-----------FREQUENCY DIVIDER FOR 10 SEC CLOCK-------------

process(clk, fast)

variable divider : integer range 0 to 100000000; -- Adjusted for 10 seconds begin

if (rising_edge(clk)) then divider := divider + 1;

if (divider = 100000000) then -- 10-second period (assuming a 100 MHz input clock) fast <= not fast;

divider := 0; end if;

end if; end process;

------ANODE COUNTER:---------------------

Process(slow) begin

if(slow'event and slow='1') then Temp <= Temp+1;

end if;

end process;

------SEVEN SEGMENT COUNTER:---------------------

Process(slow) begin

if(slow'event and slow='1') then sevensegcounter <= sevensegcounter+1; end if;

end process;

------7 SEGMENT DISPLAY SELECTION:---------------------

with Temp Select

Anode <="11111110" WHEN "0000",

"11111101" WHEN "0001",

"11111011" WHEN "0010",

"11110111" WHEN "0011",

"11101111" WHEN "0100",

"11011111" WHEN "0101",

"10111111" WHEN "0110",

"01111111" WHEN "0111",

"11111111" WHEN OTHERS;

------DECIMAL CONVERSION:---------------------

y_SCdecimal <= to_integer(unsigned(Set_clear)); y_SRdecimal <= to_integer(unsigned(Shift_rotate)); y_logicdecimal <= to_integer(unsigned(logic)); y_arithdecimal <= to_integer(signed(Arith));

------MUX:---------------------

WITH Sel(4 DOWNTO 3) SELECT

ydigit1 <= y_logicdecimal mod 10 WHEN "00",

y_arithdecimal mod 10 WHEN "01",

y_SRdecimal mod 10 WHEN "10",

y_SCdecimal mod 10 WHEN OTHERS;

WITH Sel(4 DOWNTO 3) SELECT

ydigit2 <= y_logicdecimal/10 mod 10 WHEN "00",

y_arithdecimal/10 mod 10 WHEN "01",

y_SRdecimal/10 mod 10 WHEN "10", y_SCdecimal/10 mod 10 WHEN OTHERS;

WITH Sel(4 DOWNTO 3) SELECT

ydigit3 <= y_logicdecimal/100 mod 10 WHEN "00",

y_arithdecimal/100 mod 10 WHEN "01",

y_SRdecimal/100 mod 10 WHEN "10", y_SCdecimal/100 mod 10 WHEN OTHERS;

------RECONVERTING PROCESS:---------------------

ydigit1conv <= std_logic_vector(to_unsigned(ydigit1,ydigit1conv'length));

ydigit2conv <= std_logic_vector(to_unsigned(ydigit2,ydigit2conv'length)); ydigit3conv <= std_logic_vector(to_unsigned(ydigit3,ydigit3conv'length));

------7 SEGMENT DISPLAY:---------------------

WITH ydigit1conv SELECT

y1<= "11111001" when "0001",

"10100100" when "0010",

"10110000" when "0011",

"10011001" when "0100",

"10010010" when "0101",

"10000010" when "0110",

"11111000" when "0111",

"10000000" when "1000",

"10010000" when "1001",

"11000000" when "0000",

"11111111" when others;

WITH ydigit2conv SELECT

y2<= "11111001" when "0001",

"10100100" when "0010",

"10110000" when "0011",

"10011001" when "0100",

"10010010" when "0101",

"10000010" when "0110",

"11111000" when "0111",

"10000000" when "1000",

"10010000" when "1001",

"11000000" when "0000",

"11111111" when others;

WITH ydigit3conv SELECT

y3<= "11111001" when "0001",

"10100100" when "0010",

"10110000" when "0011",

"10011001" when "0100",

"10010010" when "0101",

"10000010" when "0110",

"11111000" when "0111",

"10000000" when "1000",

"10010000" when "1001",

"11000000" when "0000",

"11111111" when others; WITH sevensegcounter SELECT

Display<="11111000" WHEN "0000", ---0

"10110000" WHEN "0001", ---7

"11000000" WHEN "0010", ---O

"10101011" WHEN "0011", ---N

"11000111" WHEN "0100", ---L

"11000111" WHEN "0101", ---L

"11000000" WHEN "0110", ---0

"10101111" WHEN "0111", ---R

"11111111" WHEN OTHERS;

end Architecture USAMA;--

TEST BENCH:

LIBRARY ieee;

USE ieee.std_logic_1164.ALL; use ieee.std_logic_unsigned.all; USE ieee.numeric_std.ALL;

ENTITY testbenchpbl IS END testbenchpbl; ARCHITECTURE 20ES070 OF testbenchpbl IS

-- Component Declaration for the Unit Under Test (UUT) COMPONENT PBL_070

PORT(

in1 : IN std_logic_vector(3 downto 0); in2 : IN std_logic_vector(3 downto 0); Sel : IN std_logic_vector(4 downto 0); clk : IN std_logic;

Anode : OUT std_logic_vector(7 downto 0); Display : OUT std_logic_vector(7 downto 0)

);

END COMPONENT;

--Inputs

signal in1 : std_logic_vector(3 downto 0) := (others => '0'); signal in2 : std_logic_vector(3 downto 0) := (others => '0'); signal Sel : std_logic_vector(4 downto 0) := (others => '0'); signal clk : std_logic := '0';

--Outputs

signal Anode : std_logic_vector(7 downto 0); signal Display : std_logic_vector(7 downto 0); BEGIN

-- Instantiate the Unit Under Test (UUT) uut: 20ES070 PORT MAP (

in1 => in1, in2 => in2, Sel => Sel, clk => clk,

Anode => Anode, Display => Display

);

-- Clock process definitions clk_process :process

begin

end process;

clk <= '1';

wait for 1 ps; clk <= '0';

wait for 1 ps;

-- Stimulus process stim_proc: process

begin

end process; END;

in1<="0000"; in2<="0000"; Sel<="00000";

for i in 0 to 32 loop Sel<=Sel+1;

end loop;

wait for 2 ps;

SIMULATION:

UCF FILE:

HARDWARE: